Object Type:    table2D

Description:    A two-dimensional table with interpolation.

Author:         U. S. Bhalla MSSM May/93

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	[in src/olf/olf_struct.h]

Size:           124 bytes (minimum)

Fields:         input       x value input to table

input2      yvalue input to table

output      Output of table lookup

negoutput   Negative of table output

table       2d interpol struct for table

alloced     Internal flag for table allocation

step_mode   Not implemented

stepsize    Not implemented

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	[in src/olf/table2d.c]

Classes:        segment

Actions:        SET  TABCREATE  PROCESS  RESET

Messages:       INPUT  x-value-for-lookup  y-value-for-lookup

SUM    value-to-sum-onto-output

PRD    value-to-multiply-output-by

X      x-value-for-lookup

Y      y-value-for-lookup

------------------------------------------------------------------------------

Notes:          TABCREATE must be called to allocate the interpol_struct

of the table.  The syntax is:

call table-element TABCREATE xdivs xmin xmax [ydivs ymin ymax]

As with other interpol_structs, xdivs and ydivs refer to the

number of divisions in the table, with the first index running

from 0 to xdivs and the second one from 0 to ydivs.  If the

ydivs, ymin, and ymax parameters are not given, these will be

set to the values used for the x variable.

By default, objects that use 2-D tables (tab2Dchannel,

table2D, and tabcurrent) have the calc_mode for the tables set

to LIN_INTERP (a pre-defined global variable equal to one), so

that linear interpolation is performed at run time, allowing

the use of a smaller table.  In order to disable

interpolation, set the calc_mode to NO_INTERP (zero).  For

example:

setfield my2Dtable table->calc_mode {NO_INTERP}

Example:        genesis > create table2D /foo

genesis > call /foo TABCREATE 5 0 1 5 0 1

genesis > setfield /foo table->table[0][0] 5

genesis > setfield /foo table->table[1][1] 10

genesis > setfield /foo table->table[3][3] 105

genesis > echo {getfield /foo table->table[3][3]}

105

See also:	table, Interpol documentation (Tables).