The following are the cell names and cell descriptions of sample

layouts designed at NIST for micromechanical designs.

1.  open-80x80

A MEMS test structure to check the progress of the EDP etchant

on an intended 80 um x 80 um cavity.  This design consists of a

single open tile.

2.  open-oxide-80x80

An additional MEMS test structure to check the progress of

the etchant on an intended 80 um x 80 um cavity with a

trampoline-like suspended oxide.  This design consists of

four trapezoidal-shaped open tiles brought together in close

vicinity such that the four cavities merge in the

post-processing etch step creating one large 80 um x 80 um

cavity with a suspended layer of oxide (see fig. 1).

3.  thermal-converter

A polysilicon (poly1) heater with a metal1/poly1 thermopile

used as an ac power sensor.

4.  thermal-actuator

A cantilever structure consisting of poly1 with metal1 on

top.  These are connected at the end of the `diving board'

and, when heated, the diving board bends upward due to the

different thermal expansion coefficients of poly1 and

metal1.

5.  pixel-80x80

A heating element with an 80 um x 80 um cavity.  The heating

element is made of meandering polysilicon (poly1) and is

suspended over top of the cavity with its encompassing oxide

after the anisotropic etch.

6.  pixel-160x160

Similar to the above structure but with a 160 um x 160 um

cavity.

7.  micro-hot-plate

The microhotplate design consists of:

a.  A meandering polysilicon (poly1) heating element with

two external connections.

b.  An aluminum (metal1) plate with four external connections

over top of the polysilicon (poly1) heating element.

There is an oxide separating the poly1 and metal1.

The heat is uniformly distributed across this metal1

plate with the metal1 resistance changing linearly with

temperature.  The metal1 sheet resistance at the

various temperatures can be obtained with a

van der Pauw measurement.

8.  gas-sensor

The gas sensor design consists of the microhotplate design

above with the addition of two layers:

a.  Four metal2 pads placed in the corners of the metal1

microhotplate (but not in contact with it). There

are four external metal2 connections.

b.  Overglass openings centered on top of the metal2 pads.

The metal2 pads and overglass openings allow a substance

(e.g., tin oxide and palladium) to be placed on top of the

whole pixel.  When heated, the resistance of the substance

changes in the presense of certain gases.  The metal2 acts

as a van der Pauw sheet resistor so the resistance change

can be measured.  This application of the microhotplate is

called a gas sensor.  (NIST has a patent pending on this

gas sensor design.)